1.测试场景构建

What is SDF SDF worlds — Gazebo harmonic documentation

SDFormat (Simulation Description Format), sometimes abbreviated as SDF, is an XML format that describes objects and environments for robot simulators, visualization, and control.

ros2 launch ros_gz_sim gz_sim.launch.py gz_args:=empty.sdf

测试模型添加SDF worlds — Gazebo harmonic documentation

ros2 launch ros_gz_sim gz_spawn_model.launch.py world:=empty file:=$(ros2 pkg prefix --share ros_gz_sim_demos)/models/vehicle/model.sdf entity_name:=my_vehicle x:=5.0 y:=5.0 z:=0.5

2.开源测试模型

Gazebo - Walking actor

Gazebo - Ignition ROS 2 CI

Gazebo - Prius on Sonoma Raceway

Gazebo - Edifice demo

3.ROS&GAZEBO联合调试

Use ROS 2 to interact with Gazebo — Gazebo harmonic documentation

ROS和Gazebo可通过ros_gz_bridge通信的消息种类：

ros_gz/ros_gz_bridge/README.md at jazzy · gazebosim/ros_gz

We can initialize a bidirectional bridge so we can have ROS as the publisher and Gazebo as the subscriber or vice versa. The syntax is /TOPIC@ROS_MSG@GZ_MSG, such that TOPIC is the Gazebo internal topic, ROS_MSG is the ROS message type for this topic, and GZ_MSG is the Gazebo message type.

案例：\ros\jazzy\share\ros_gz_sim_demos\launch  “battery.launch.py”Line 51

# Bridge

bridge = Node(

package='ros_gz_bridge',

executable='parameter_bridge',

arguments=[

'/model/vehicle_blue/cmd_vel@geometry_msgs/msg/Twist@gz.msgs.Twist',

'/model/vehicle_blue/battery/linear_battery/state@sensor_msgs/msg/BatteryState@'

'gz.msgs.BatteryState'

],

output='screen'

)

ros_gz/ros_gz_bridge/README.md at jazzy · gazebosim/ros_gz

4.GAZEBO robot 模型构建

Building your own robot — Gazebo harmonic documentation

Link for calculate mass and inertia: Mass Moment of Inertia Calculator

Gazebo topic info send 7& subscribe test:

More command of Gazebo transport.Gazebo Transport: Tutorials

Appendix A Useful plugin of gz

Triggered Publisher: gz-sim/tutorials/triggered_publisher.md at gz-sim7 · gazebosim/gz-sim

KeyPublisher